Lab Exercise 3

IT4030 - Internet of Things (IoT)                                                                 2024

Lab 3: Using a Light Dependent Resistor (LDR) with an Arduino

Objectives:

- Studying about the Ohm’s Law, resistors, and voltage divider rule

- Creating a circuit in Tinkercad to get the readings from the Light Dependent Resistor (LDR) using the Arduino

Background:

Ohm’s law states that the voltage across a conductor is directly proportional to the current flowing through it, provided all physical conditions and temperature remain constant. The mathematical equation that describes this relationship is as follows.

𝑽 = 𝑰 𝑹

V = Voltage (V)

B

Sc in

Information Technology

Year 4

-

Semester 1

/2

I = Current (A)

R = Resistance (𝞨)

A resistor is a passive two-terminal electrical component that limits the current that flows through a circuit. The symbol of a resistor and resistors with different resistance values are shown in Figure 1 and Figure 2 respectively.

Figure 1: Symbol of a resistor

Figure 2: Different resistors

1

The resistor color code chart in Figure 3 shows how to determine the resistance and tolerance of the resistors. This can also be used to specify the color of the bands when the values are known.

Figure 3: The resistor color code chart

In electronics, the voltage divider circuit (potential divider) which is depicted in Figure 4, is a passive linear circuit that produces an output voltage (Vout) as a fraction of its input voltage

(Vin).

Figure 4: Voltage divider circuit

A Light Dependent Resistor (LDR) which is also known as a photoresistor is a passive component that decreases its resistance with respect to the luminosity (light) that falls onto the component's sensitive surface.  An LDR is shown in Figure 5.

Figure 5: An LDR

Procedure:

- Using Tinkercad, create the following circuit to get the readings from an LDR using the Arduino.

Circuit

Code

int value = 0;

void setup()

{

pinMode(A0, INPUT);

pinMode(9, OUTPUT);

Serial.begin(9600);

}

void loop()

{

// read the value from the LDR

value = analogRead(A0);

// print the LDR reading

Serial.println(value);

// map the reading to a range for the LED

analogWrite(9, map(value, 0, 1023, 0, 255));

delay(100); // Wait for 100 millisecond(s)

}

Video