Lab Exercise 4

IT4030 - Internet of Things (IoT)                                                                 2024

Lab 4: measure humidity and temperature using a NodeMCU (ESP8266) and a DHT11 sensor while publishing the data to Adafruit IO

B

Sc in

Information Technology

Year 4

-

Semester 1

/2

- Goto Adafruit and create a user account. (IO -> Feeds{Create two feeds})

Adafruit Industries, Unique & fun DIY electronics and kits

- i)Search “Nodemcu ESP8266 Preferences link” and copy the link or,

ii)visit Quick Start to Nodemcu (ESP8266) on Arduino IDE : 3 Steps (with   Pictures) - Instructables or any suitable website and copy Preferences link or,

iii)use http://arduino.esp8266.com/stable/package_esp8266com_index.json

link as Preferences link.

- Goto Arduino IDE -> File -> Preferences -> Additional boards manager URLs ->

Paste the link -> OK

- Arduino IDE -> Tools -> Board -> Board Manager -> Search “esp8266” -> Install

- Arduino IDE -> Tools -> Board -> esp8266 -> Generic ESP8266 Module

- Code

#include <ESP8266WiFi.h>

#include <Adafruit_MQTT.h>

#include <Adafruit_MQTT_Client.h>

#include <DHT11.h>

// Replace these with your network and Adafruit IO credentials

#define WIFI_SSID       "wifiname"

#define WIFI_PASS       "password"

#define AIO_SERVER      "io.adafruit.com"

#define AIO_SERVERPORT  1883                   // Use 8883 for SSL

#define AIO_USERNAME    "hanojhanr"

#define AIO_KEY         "aio_iRMM32P9cjTIzez13nlulpmfN0qj"

#define DHTPIN 4    // DHT11 sensor data pin connected to ESP8266 to pin 4

DHT11 dht11(DHTPIN);

WiFiClient client;

Adafruit_MQTT_Client mqtt(&client, AIO_SERVER, AIO_SERVERPORT, AIO_USERNAME, AIO_KEY);

// Setup feeds for temperature and humidity

Adafruit_MQTT_Publish temperatureFeed = Adafruit_MQTT_Publish(&mqtt, AIO_USERNAME "/feeds/temperature");

Adafruit_MQTT_Publish humidityFeed = Adafruit_MQTT_Publish(&mqtt, AIO_USERNAME "/feeds/humidity");

void setup() {

Serial.begin(115200);

delay(10);

// Connect to Wi-Fi

Serial.println(); Serial.println();

Serial.print("Connecting to ");

Serial.println(WIFI_SSID);

WiFi.begin(WIFI_SSID, WIFI_PASS);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println();

Serial.println("WiFi connected");

Serial.println("IP address: "); Serial.println(WiFi.localIP());

// Connect to Adafruit IO

connectToAdafruitIO();

// Initialize the DHT sensor

// dht11.begin();  // Not needed for DHT11 library

}

void loop() {

// Ensure the connection to Adafruit IO is maintained

if (mqtt.connected()) {

mqtt.processPackets(10000);

mqtt.ping();

} else {

connectToAdafruitIO();

}

int temperature = 0;

int humidity = 0;

// Read temperature and humidity from DHT11

int result = dht11.readTemperatureHumidity(temperature, humidity);

// Check if any reads failed

if (result == 0) {

// Publish the data

Serial.print("Publishing temperature: "); Serial.println(temperature);

if (!temperatureFeed.publish(temperature)) {

Serial.println("Failed to publish temperature");

}

Serial.print("Publishing humidity: "); Serial.println(humidity);

if (!humidityFeed.publish(humidity)) {

Serial.println("Failed to publish humidity");

}

} else {

Serial.print("Failed to read from DHT sensor! Error: ");

Serial.println(DHT11::getErrorString(result));

}

// Wait for a few seconds before sending the next reading

delay(2000);

}

void connectToAdafruitIO() {

int8_t ret;

// Attempt to connect to Adafruit IO

Serial.print("Connecting to Adafruit IO... ");

while ((ret = mqtt.connect()) != 0) {

Serial.println(mqtt.connectErrorString(ret));

Serial.println("Retrying in 5 seconds...");

mqtt.disconnect();

delay(1000);

}

Serial.println("Adafruit IO connected!");

}

- Arduino IDE -> Tools -> Select correct Port (If port is not detected, download and install CH340 driver. Use this link CH340 Drivers for Windows, Mac and Linux (gogo.co.nz) or Google and download)

- Arduino IDE -> Tools -> Manage Libraries… -> Search and Install ESP8266WiFi, Adafruit_MQTT and DHT11

- Change WIFI_SSID and WIFI_PASS in your code to your Wi-Fi or mobile hotspot (Only Android works) ID and Password.

- Goto Adafruit -> IO -> Click API Key

- Change AIO_USERNAME and AIO_KEY in your code to your Adafruit Username and Active Key.

- Tools -> Serial Monitor -> Change to 115200baud

- Upload the code to NodeMCU and monitor the output from Serial Monitor and readings for temperature and humidity will be printed, and the data will be published to Adafruit IO.

- We can use NodeMCU without connecting to the computer.

- NodeMCU is connecting to a powerbank.

DHT11 -> NodeMCU

+ -> 3V

- -> G

Out -> D2